AIMS5790(Class Section Code)

AIMS5790 Artificial Intelligence Project I

Data-driven Wind Power Forecasting with Weather Foundation Models

BY

FAN Zehui

1155184696

Master of Science in Artificial Intelligence programme

Faculty of Engineering

The Chinese University of Hong Kong

04, 2026

Abstract

Accurate short-term wind power forecasting is essential for grid stability and operational planning, yet remains challenging due to the complex interactions between atmospheric conditions and turbine dynamics. However, existing methods fail to effectively incorporate weather forecasting with wind turbine data (i.e., SCADA), leading to suboptimal solutions. To address this, we introduce a multimodal framework that integrates historical point-based SCADA data with grid-based Numerical Weather Prediction (NWP) forecasts, which is challenging due to heterogeneous input and the complex physical wind-turbine interactions. Our approach first explicitly decomposes inputs into scalar and vector features to better capture both site-specific and geometric dependencies and then incorporates a geometric encoder to extract rotation-invariant features from wind vectors. We further leverage a Fourier Neural Operator (FNO) architecture, which performs global convolutions in the frequency domain to efficiently model long-range spatiotemporal relationships. Extensive experiments on three real-world wind farms, with weather forecasting data, demonstrate that our model consistently outperforms state-of-the-art baselines, highlighting the effectiveness of its physically informed design.

- Introduction

Global electricity consumption is entering a period of strong growth, as electrification expands in buildings, transport and industry, while additional demand is increasingly driven by cooling and data centers. Wind power, being a clean and renewable energy source is considered a key low-emissions technology and a valid solution in this transition . In particular, the IEA projects global wind generation to grow at about 11% per year on average during 2025–2027, accounting for roughly one-third of the additional global electricity demand over the same period  . However, wind power output, i.e., SCADA data, is strongly conditioned on atmospheric variability and local flow conditions, which leads to pronounced uncertainty and temporal intermittency in electricity production  . Such variability complicates system operation because power grids require continuous balancing between supply and demand, motivating accurate short-term wind power forecasting to improve dispatch, reserve scheduling and reliability . Despite its practical benefits, wind power forecasting remains challenging due to the chaotic nature of atmospheric dynamics and the complex turbine–flow interactions within wind farms .

Existing approaches can be classified into physical and data-driven models. Physical models solve equations that incorporate Numerical Weather Prediction (NWP), turbine parameters, and farm geography to predict wind power. Nevertheless, their accuracy is limited by the oversimplified parametrization of complex physical processes and the high demands of computational resources. In contrast, data-driven models directly learn neural networks to approximate the wind power outputs, which can be broadly divided into two types: (1) Spatio-temporal forecasting. They integrate both spatial and temporal dependencies to enhance prediction accuracy by capturing how wind fields evolve over time and interact across geographical regions. A common architecture is Graph Neural Networks , where they typically represent turbines as nodes, construct edges among them via distance, and perform message passing thereon. While these models are effective for exploiting cross-turbine correlations from SCADA histories, they are often deployed in a SCADA-only setting and thus may miss upcoming synoptic/mesoscale changes that are better captured by NWP ; (2) Wind power scenario generation. Instead of outputting a single point estimate, probabilistic methods aim to produce calibrated predictive distributions or multiple coherent future trajectories (scenarios), which are crucial for risk-aware decision making in markets and system operation. A classical NWP-based approach is EMOS , which statistically calibrates ensemble forecasts by fitting a parametric predictive distribution to correct bias and dispersion. More recently, diffusion-based generative models have been developed for probabilistic time-series forecasting by learning a denoising process that can sample diverse yet coherent trajectories . In the wind power domain, conditional latent diffusion models have been proposed to generate short-term wind power scenarios with improved sharpness and calibration under complex uncertainty .Although these NWP-driven probabilistic approaches can quantify uncertainty, many of them do not explicitly model turbine operational states and the non-stationary turbine–wind relationship, which reduces forecasting accuracy.

However, it is not straightforward to incorporate data-driven wind power forecasting with numerical weather prediction due to:

- Data heterogeneity. Insufficient resolution in NWP grids prevents direct access to wind conditions at individual turbine locations, presenting a fundamental data heterogeneity challenge. The grid-point mismatch leads to information loss and increased modeling complexity. It obscures the precise local wind conditions that directly drive each turbine, forcing the model to infer these conditions from coarse surrounding data.

- Physical interaction. Wind power generation is fundamentally dominated by wind speed magnitude and wind direction. The misalignment between wind direction and nacelle yaw directly influences effective inflow and thus power conversion efficiency. Therefore, it is necessary to capture the physical interactions between gridded wind fields and point-level inflow, as well as the directional effects introduced by turbine layout and flow dynamics.

To address the above issue, we present a novel multimodal forecasting framework that integrates geometric priors and spectral feature fusion for short-term wind power prediction. We first employ a geometric encoder to explicitly model directional inductive biases by transforming raw wind vectors and nacelle angles into rotation-invariant scalar features, improving its generalization across diverse wind regimes. Fourier Neural Operator (FNO) is flexible to adapt to varying resolutions. We then employ an FNO-based Fusion to effectively combine the encoded turbine states with gridded weather forecasts by performing global convolutions in the frequency domain, thereby capturing complex, long-range spatiotemporal dependencies within and across modalities. Extensive experiments on three real-world wind farms and weather reanalysis data demonstrate that our framework consistently outperforms a comprehensive range of both spatio-temporal and wind power scenario generation models. Ablation studies further validate the contribution of each core component, confirming the necessity of both the geometric encoding and the neural operator fusion.

- Related Work

  - Spatio-temporal wind power forecasting

Deterministic wind power forecasting (WPF) has evolved into a sophisticated spatio-temporal point estimation task, shifting the focus from isolated time-series modeling to the integrated representation of regional meteorological systems . Within this paradigm, algorithms are trained to map historical SCADA-derived observations onto discrete future power sequences by minimizing distance-based loss functions . The methodology for learning spatio-temporal dependencies is primarily centered on the following technical frameworks: Spatial Interaction via Graph Neural Networks (GNNs): Recognizing that wind farms possess regional characteristics where turbines interact with one another through air movement, current research utilizes GNNs to model these complex dependencies . By treating turbines or farms as nodes and their geographical or climatic relationships as edges, GNNs effectively capture regional interactions that single-site models fail to perceive, thereby enhancing the accuracy of the forecasting system. Temporal Paradigms (AR vs. NAR): The temporal dimension is processed through either Autoregressive (AR) or Non-autoregressive (NAR) architectures. AR models, such as LSTM  and GRU , iteratively propagate hidden states to preserve the continuity of historical trends, demonstrating high robustness in short-term horizons. Conversely, NAR models—including CNN-based (TCN ), Transformer-based (Informer , Autoformer ), and MLP-based (TSMixer ) architectures—generate the entire forecast sequence in a single pass. This generative approach leverages global dependencies more effectively, mitigates the accumulation of errors inherent in iterative methods, and increasingly matches or exceeds AR performance in long-term forecasting.

  - Wind power scenario generation

In the current research landscape, wind power scenario generation is predominantly conducted within a probabilistic forecasting framework aimed at capturing the inherent stochasticity and meteorological volatility of wind energy to enhance grid reliability . Methodologically, this task falls into three main streams: probability distribution estimation (e.g., DeepAR ), which predicts parametric distribution parameters such as mean and variance via likelihood-based learning to enable trajectory sampling; quantile regression (e.g., QR-LSTM), a non-parametric approach that estimates conditional quantiles without assuming a specific distribution by minimizing the pinball loss across multiple quantile levels; and Bayesian neural networks (e.g., Bayesian-LSTM), which inject uncertainty directly into model weights through Bayesian inference—often approximated via variational methods or Monte Carlo dropout—to produce ensembles of diverse, risk-aware scenarios . Architecturally, autoregressive models like LSTM and GRU are favored for their ability to preserve temporal dependencies and trends through iterative information propagation. Scenario quality is rigorously evaluated using a suite of metrics, including the Continuous Ranked Probability Score (CRPS) for overall distributional accuracy, and the Prediction Interval Coverage Probability (PICP) and Prediction Interval Normalized Average Width (PINAW) to assess the balance between interval reliability and sharpness .

Despite these advances, most existing methods still rely heavily on GNN and time series models that primarily use SCADA data and exhibit insufficient attention to spatiotemporal dependencies across wind farms. Meteorological data, such as numerical weather predictions, are typically incorporated through basic feature fusion approaches that do not adequately capture the complex spatial patterns, directional movements, or nonlinear evolution of weather systems . As a result, the generated scenarios often miss important atmospheric dynamics, revealing a clear need for models that better exploit the rich spatiotemporal structure in weather data.

- Problem Definition

We formulate the short-term wind power forecasting task in a multi-modal setting that integrates historical turbine-level SCADA data and grid-based Numerical Weather Prediction (NWP) forecasts. The input features are decomposed into scalar and vector quantities to explicitly distinguish between geometric-agnostic and geometric variables. Let a wind farm consist of N turbines. For each turbine i at historical time step t∈{1,…,L}, we observe turbine scalar features hit∈RDh (e.g., wind speed magnitude, power output, operational status) and turbine vector features qit∈RDq (primarily the 2D wind vector). At the forecast anchor time t=L, the NWP system issues predictions for future lead times τ=1,…,T. For each τ, it provides a gridded forecast over the farm region, including scalar fields gL+τs∈RH×W×Vs (e.g., temperature, pressure) and vector fields gL+τv∈RH×W×Vv (e.g., zonal and meridional wind components), where H and W denote the spatial grid dimensions, and Vs, Vv are the numbers of scalar and vector variables, respectively. The objective is to predict the hourly wind power output for all N turbines over the next T hours. Formally, we learn a mapping fθ:

yL+1:L+T=fθ{ht,qt}t=1L; {gL+τs,gL+τv}τ=1T,

where yL+1:L+T∈RN×T denotes the predicted power sequence.

- Method

  - Overall Framework

Our model follows an encoder–decoder design. A multi-modal fusion encoder maps heterogeneous inputs (SCADA point observations, CERRA analysis grids, and CERRA forecast grids) into: (i) a history latent sequence Z∈RL×N×D and (ii) lead-wise exogenous features Efc∈RH×N×E. A graph recurrent decoder then predicts future power Yt+1:t+H conditioned on Z,Efc.

  - Input Construction and Normalization

We align SCADA to hourly targets and 3-hour inputs. Wind direction θ and nacelle yaw ψ are converted into sin/cos pairs to avoid angle discontinuity. All scalar inputs are standardized (z-score) using the training period and valid entries. CERRA forecast features are normalized separately with per-dimension statistics. Invalid entries are set to zero (with corresponding validity masks for grid features).

  - Multi-Modal Spatio-Temporal Encoding

SCADA functional representation (FuncRepVFR). The SCADA streams are irregularly missing and exhibit both temporal persistence and spatial correlation across turbines. To robustly encode such data, we adopt a functional representation that treats each (time, turbine) pair as a point in a shared spatio-temporal space. This design enables the encoder to (i) exchange information across turbines and time steps in a permutation-invariant manner, and (ii) remain compatible with masking (missing values are set to zero and excluded by the valid mask in training/evaluation). Concretely, the SCADA inputs are first projected into a latent space, then processed by a set-based spatio-temporal convolution (SetConv) followed by a temporal spectral refinement layer (Neural Fourier Layer). FuncRepVFR is applied to the SCADA scalar channels (power-related features and wind speed) and also to the SCADA direction-related channels (wind vector and alignment cues).

- SetConv (spatio-temporal kernel mixing). SetConv builds a soft neighborhood for all (time, turbine) points using their normalized spatio-temporal coordinates and performs kernel smoothing on the latent features. The core operation is a Gaussian kernel followed by row-normalized aggregation:

Kpq=exp​-12(tp-tqlt)2-12(xp-xqls)2-12(yp-yqls)2,

hp=q​Kpqj​Kpj+ϵ hq,

where hp is the projected latent at point p (a specific time–turbine pair), tp,xp,yp is its normalized coordinate, lt and ls are learnable temporal/spatial length-scales, and ϵ is a small constant for numerical stability. We optionally keep only the top-k kernel weights per row to reduce noise and improve efficiency.

- Neural Fourier Layer (temporal spectral refinement). After SetConv, we refine each turbine’s latent sequence by operating in the frequency domain to better capture periodicity and multi-scale temporal patterns. We apply a real FFT along the history axis, learn a complex linear transformation on the lowest-frequency modes, and add it back as a residual in the spectrum:

hf=rFFThf,  h'f=hf+gf⋅Wf hf,  h'=irFFTh'

where f is a frequency index, Wf is a learnable (complex-valued) linear map for mode f, and gf is a learnable per-mode gain (with optional spectral dropout). Finally, we apply a short temporal convolution and layer normalization to obtain the SCADA latent representation (shape: batch × history × turbines × embedding).

Grid encoding (GridEncoder). The meteorological products (CERRA analysis and CERRA forecast) are represented as a set of neighboring grid points around each turbine. A grid encoder should (i) exploit the structured local spatial context around each turbine, (ii) handle missing grid values, and (iii) aggregate the K neighbors into a turbine-level representation while respecting distance and relevance. We implement this with (a) per-point feature projection, (b) relative-position embedding, (c) optional set-convolution over the K local points, and (d) attention-based aggregation with an explicit distance penalty.

Specifically, each grid point is embedded as:

pk=Wx xk+ϕΔxk,Δyk,dk,

where xk is the grid feature vector at neighbor k (e.g., wind speed for the scalar encoder or u,v for the vector encoder), Δxk,Δyk,dk is the normalized relative position (including distance) from the turbine to that grid point, Wx is a learnable projection, and ϕ⋅ is a small MLP. We then compute an attention score for each neighbor and aggregate:

ak=ψpk-λd dk,  wk=softmax{aj}j=1K,  z=k=1Kwk pk,

where ψ⋅ is an MLP producing a scalar score, and λd is a learnable distance-penalty strength. This outputs a turbine-level grid latent (shape: batch × time × turbines × embedding), used for both CERRA analysis (history-aligned) and CERRA forecast (lead-time aligned).

Horizon pooling for forecast embeddings.The CERRA forecast branch produces a lead-wise embedding sequence. To provide a compact, time-invariant exogenous summary for decoding, we pool the lead dimension by attention and then project to the model embedding size:

ατ=softmax​{seτ}τ=1H,  zfc=LN​Wpτ=1Hατeτ,

where eτ is the forecast embedding at lead τ, s⋅ is a scoring MLP, Wp is a projection, and LN denotes layer normalization. We broadcast zfc across the history axis when needed for fusion, while also keeping the original lead-wise embeddings as decoder exogenous inputs.

  - Direction-Aware Geometric Modeling

Wind vector construction and nacelle misalignment. Wind is fundamentally a vector field, and directional effects (advection and wakes) are essential for modeling turbine interactions. Moreover, turbine operation is defined in a turbine-centric frame (nacelle yaw), so the relative direction between incoming wind and nacelle orientation is a physically meaningful cue. To avoid angle discontinuities, we construct vectors and relative angles using continuous sine/cosine forms.

We compute the earth-frame horizontal wind vector from SCADA wind speed and direction as:

u=-Wsinθ,  v=-Wcosθ,

where W is wind speed and θ is wind direction (represented by sine/cosine in the data pipeline). We also compute the nacelle–wind misalignment using a continuous representation:

cosΔψ=cosθcosψ+sinθsinψ,  sinΔψ=sinθcosψ-cosθsinψ,

where ψ is nacelle direction and Δψ=θ-ψ is the misalignment angle. These misalignment cues are fed into the SCADA direction-related branch and also used as physics contexts for fusion.

Physics alignment contexts for fusion. To explicitly encode consistency between measured inflow and meteorological products, we compute alignment features between two vectors using normalized dot/cross products. For example, the alignment between SCADA inflow usc,vsc and analysis inflow uan,van is:

cosδ=uscuan+vscvanusc2+vsc2+ϵuan2+van2+ϵ,

sinδ=uscvan-vscuanusc2+vsc2+ϵuan2+van2+ϵ,

where δ is the signed directional difference and ϵ stabilizes normalization. We use analogous alignment features for analysis–forecast consistency, and inject these physics contexts into the scalar fusion gate (Section 3.9).

Wake-directed cross-turbine mixing. Turbines do not evolve independently: upstream turbines can affect downstream turbines via advection and wake deficits. To model directional inter-turbine influence without changing the dataloader, we construct a soft upstream-to-downstream mixing operator using the turbine layout and the farm-level flow direction estimated from SCADA wind vectors. We compute along-flow and cross-flow distances between turbines and build a directional weight matrix, then perform message passing on SCADA latents (residual + normalization).

The core weighting uses a soft upstream indicator and Gaussian decay:

dij∥=xi-xju+yi-yjv,  dij⊥=xi-xjv-yi-yju,

wij=σ​s⋅dij∥τ⋅exp​-(maxdij∥,0σ∥)2⋅exp​-(dij⊥σ⊥)2,

zi'=LN​zi+ηj​wijzj,

where xi,yi is the turbine layout coordinate, u,v is the unit flow direction, σ⋅ is the sigmoid function, wij is row-normalized wij, and τ,σ∥,σ⊥,s,η are learnable scalars controlling directionality and mixing strength. We additionally expose summary features (e.g., total upstream weight and mean along-flow distance) as inputs to the fusion contexts.

Downwind grid-point penalty. Inflow-relevant grid points should be emphasized when aggregating local meteorological fields. We implement this by modifying the distance feature used by the grid-attention mechanism: grid points on the downwind side receive a larger effective distance (thus lower attention), and the penalty strength/direction is learnable to avoid coordinate-sign ambiguity.

The modified distance is:

d'=d+λ ReLUs⋅Δx u+Δy v,

where Δx,Δy,d is the normalized relative position of a grid point, u,v is the unit wind direction estimated from CERRA u/v, and λ,s are learnable scalars. We then replace d by d' in the distance-penalized attention of GridEncoder.

  - Frequency-Domain Tri-Branch Fusion with Vector Modulation

Tri-branch fusion in the frequency domain. We maintain three information sources aligned to the history axis: SCADA observation, CERRA analysis, and CERRA forecast summary. A naive concatenation can be unstable because the three sources have different noise levels and temporal characteristics. We therefore fuse them in the frequency domain: we first transform each latent sequence along time to obtain a complex spectrum, estimate frequency-wise reliability via a small gating network, then mix the spectra and transform back to the time domain. This design allows the model to adaptively trust different sources at different temporal frequencies (e.g., smooth synoptic components versus short-term fluctuations).

Let the three branch latents be transformed by rFFT along time:

zrf=rFFTzrf,  mrf=log​1+meanzrf,

where r indexes the source (SCADA / analysis / forecast), f is a frequency index, and mrf is a compact magnitude descriptor. For the scalar fusion gate, we additionally compute spectral magnitudes of the physics contexts (SCADA–analysis alignment and analysis–forecast alignment) and inject them into the gate input. The gate outputs frequency-wise weights:

wf=softmax​gmscf,manf,mfcf,mctx1f,mctx2f,mmeanf,

where g⋅ is a small MLP, mmeanf is the mean magnitude across the three branches, and wf produces three weights for the three branches. We then obtain a gated spectral mixture and blend it with the simple average as a stabilizing residual path:

zself=r​ wrf zrf,  zavgf=13r​zrf,

zf=1-αzavgf+α zself,

where α is a learnable scalar mapped by a sigmoid. After inverse FFT, we apply a short temporal convolution and layer normalization to obtain the fused scalar latent. We apply the same frequency-domain fusion idea to the direction-related branch (without context injection) to obtain the fused vector latent.

Vector-to-scalar modulation (VecFiLM). Directional effects should influence how scalar signals (especially power) are interpreted. Instead of forcing the scalar fusion gate to learn all such couplings implicitly, we use the fused direction-related latent to modulate the fused scalar latent via a FiLM-like transformation. This improves stability and encourages a structured interaction between directionality and scalar dynamics.

The modulation is:

γ,β=MLPzv,  z=1+s γ⊙zs+s β,

where zs and zv are the fused scalar and fused direction-related latents, s is a learnable global scale (sigmoid-mapped), and ⊙ denotes element-wise multiplication. The final encoder output z is fed to the decoder, and the lead-wise CERRA forecast embeddings are concatenated as decoder exogenous inputs.

Graph recurrent decoder (AGCRN-style).To predict the future power trajectory, we use an encoder–decoder graph recurrent network with a learned adaptive adjacency matrix. The adaptive adjacency is computed from trainable node embeddings and normalized by a row-wise softmax. At each step, the recurrent cell updates hidden states using graph convolutions over this adjacency. The decoder is conditioned on exogenous meteorological embeddings at each lead time.

The adaptive adjacency is:

A=softmaxReLUEE⊤,

where E denotes trainable node embeddings. A GRU-like update then produces predictions iteratively across the forecast horizon.

Training objective. We minimize the masked MAE over the prediction window, which ignores periods with curtailment/downtime and missing labels:

L=τ​n​Mτ,n Yτ,n-Yτ,nτ,n​Mτ,n+ϵ,

where Mτ,n is the supervision mask, Yτ,n is the target power, and ϵ stabilizes the denominator. We report masked MAE and masked RMSE, and additionally summarize results over short (<3 hours) and longer (<6 hours) horizons by slicing the prediction window.

- Experiment

Table 1: Overall performance comparison. Best and second-best results (excluding Persistence and Historical Mean) are high-lighted in bold with orange background and underlined with gray background, respectively. Lower values are better (↓).